ML Intern Take-Home Assignment

Sensor-Based Driving Event Detection

This assignment is based on a real, common problem in mobile sensor apps: a smartphone records accelerometer and gyroscope data at high frequency (typically 50 Hz) and GPS at a much lower, often irregular rate (roughly 1 Hz, sometimes with multi-second gaps), and the app needs to automatically detect driving events in real time, on-device, as the data streams in.

This assignment checks three things: (1) how you read and label raw multi-axis sensor data by eye, (2) how comfortable you are with the math behind turning noisy signals into reliable features, and (3) how you'd design detection logic that runs live rather than on a finished file. It should take about 60–90 minutes. Please answer in a document or notebook, showing your reasoning, not just final answers.

## Phone mount and axis reference

The phone is mounted vertically (portrait orientation) on a holder on the car's front dashboard, screen facing the driver, fixed so it does not rotate during the trip. Given this mount, the sensor axes map to the vehicle as follows:

- accel_y_g — vertical axis. ≈ 1.0 g at rest (gravity); spikes or dips sharply on bumps and potholes.

- accel_x_g — lateral axis (side to side). Captures turning, swerving, and lane changes.

- accel_z_g — longitudinal axis (front to back). Captures acceleration and braking.

- gyro_x_dps — pitch rate (nose dips/lifts), e.g. during hard braking, hard acceleration, or a sharp bump (in this data, the sharpest pitch spike actually happens on the pothole, not the braking/acceleration events — don't assume the biggest signal always means the same thing).

- gyro_y_dps — yaw rate (turning left/right).

- gyro_z_dps — roll rate (side-to-side tilt), e.g. during a swerve or a sharp bump.

Units: the _g suffix is g-force (1 g ≈ 9.81 m/s², so accel values are a small fraction of 1 except the near-1.0 g baseline on the vertical axis), _dps is degrees per second, and _mps is metres per second.

The sign (positive vs. negative) of an event is not given to you in advance — use the patterns in the data, and how they line up with gps_speed_mps, to work it out.

Note: this excerpt includes two turn/lane-change moments, where accel_x (lateral) and gyro_y (yaw) actually move away from baseline — you'll see them if you look. They're still part of ordinary driving, though, not a harsh event: don't invent a separate label for them, just call that stretch Normal Driving.

## Q1. Label the sensor events (Data Labeling)

Attached is sample_sensor_data.csv: a 25.7-second, 258-data-row log (259 lines including the header) from the mount described above, sampled at 10 Hz (simplified from the real 50 Hz for this exercise). It has two timestamp columns: timestamp is the accelerometer/gyroscope reading time; gps_timestamp is the time of the last GPS fix — notice it only updates roughly once a second and holds its last value in between, which is the real-world GPS cadence this kind of app has to work around. This trip includes two occurrences each of Clutch Release, Harsh Acceleration, Harsh Braking, and Pothole/Bump — deliberately of different strength or duration, so a segment doesn't always look exactly like the last one you saw. It also includes a stretch of gentle, gradual braking down to a stop, which should NOT be labeled Harsh Braking — part of the exercise is telling the two apart. A preview of the first 14 rows (t = 0.0 to 1.3s) is below (gps_timestamp omitted here for space — it's in the full file).

| timestamp | accel_x_g | accel_y_g | accel_z_g | gyro_x_dps | gyro_y_dps | gyro_z_dps | gps_speed_mps |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10:15:00.000 | -0.001 | 1.001 | 0.013 | -0.07 | 0.02 | 0.17 | 0.0 |
| 10:15:00.100 | 0.0 | 1.003 | 0.009 | -0.81 | -0.39 | -0.82 | 0.0 |
| 10:15:00.200 | 0.009 | 1.004 | -0.014 | 0.96 | 0.93 | 0.31 | 0.0 |
| 10:15:00.300 | 0.003 | 0.993 | -0.015 | 0.06 | -0.88 | -0.62 | 0.0 |
| 10:15:00.400 | -0.008 | 0.991 | -0.001 | -0.12 | 0.68 | 0.04 | 0.0 |
| 10:15:00.500 | 0.004 | 1.0 | 0.005 | -0.09 | -0.44 | 1.0 | 0.0 |
| 10:15:00.600 | 0.015 | 1.007 | 0.006 | -0.37 | -0.54 | -0.42 | 0.0 |
| 10:15:00.700 | -0.013 | 1.005 | -0.003 | 0.69 | -0.23 | 0.92 | 0.0 |
| 10:15:00.800 | 0.01 | 0.99 | -0.009 | 0.82 | -0.06 | 0.96 | 0.0 |
| 10:15:00.900 | -0.003 | 0.991 | 0.004 | 0.56 | -0.46 | -0.83 | 0.0 |
| 10:15:01.000 | -0.047 | 1.121 | 0.031 | -10.7 | -4.57 | -9.57 | 0.0 |
| 10:15:01.100 | 0.083 | 0.916 | 0.007 | -1.47 | -5.57 | 5.57 | 0.0 |
| 10:15:01.200 | -0.103 | 1.037 | -0.046 | -2.22 | -5.17 | -5.52 | 0.0 |
| 10:15:01.300 | 0.132 | 1.079 | -0.024 | 10.78 | -5.21 | -2.54 | 0.0 |

Using the full CSV file, split the 25.7 seconds into segments and label each one with exactly one of: Stationary, Clutch Release, Normal Driving, Harsh Acceleration, Harsh Braking, Pothole/Bump. For each segment, give the approximate time range (using the timestamp column) and one sentence on which signal(s) you used to decide — e.g. “sustained negative accel_z with a matching drop in gps_speed_mps.” Note: Clutch Release is a genuinely brief, jittery, multi-axis event (short vibration burst across several axes, without a sustained directional acceleration) rather than a smooth ramp — look for that shape.

## Q2. Feature engineering (Math)

Two features commonly used to detect driving events from raw sensor streams are jerk and windowed RMS.

- Write the formula for jerk (rate of change of acceleration) given a discrete accel_z series sampled at 10 Hz, and explain in one sentence why jerk is more useful than raw acceleration for catching the onset of a harsh event.

- Write the formula for the RMS (root-mean-square) of the gyro_z_dps signal over a 0.5-second rolling window, and explain what a sustained rise in this value would suggest about the vehicle's motion.

- Using the rows where timestamp = 10:15:08.200 to 10:15:08.600 in the attached CSV (5 data points, spanning the actual onset of the first harsh braking event), compute the average jerk in accel_z across that window. Show your working. This is the same event as the first harsh braking segment you labeled in Q1, but zoomed into the moment it begins.

## Q3. Evaluating an imbalanced classifier (Statistics)

Harsh-event detection is a heavily imbalanced problem — out of 10,000 driving-second windows, only 180 are true harsh-braking events. A model is evaluated on this data and produces the following confusion matrix:

|  | Predicted: Harsh Braking | Predicted: Normal |
| --- | --- | --- |
| Actual: Harsh Braking | 144 | 36 |
| Actual: Normal Driving | 212 | 9,608 |

- Calculate precision, recall, and F1-score for the Harsh Braking class.

- Given that missing a real harsh-braking event (a false negative) is more costly for a driver-safety monitoring app than a false alarm, would you tune the model's decision threshold up or down from its current setting? Explain your reasoning in 2–3 sentences, and name one metric besides accuracy you'd track while doing this.

## Q4. Real-time detection with mismatched sensor rates (System Design)

Accelerometer and gyroscope data arrive at 50 Hz, but GPS fixes on a phone can arrive irregularly — sometimes every 1 second, sometimes with 4–5 second gaps depending on signal conditions. Detection has to happen live, as each new sensor sample arrives, not after the trip is over — a harsh-acceleration or clutch-release detector that depends on both the accelerometer/gyroscope (to confirm the motion pattern) and GPS (to confirm the speed actually changed) starts to miss real events whenever GPS gaps get large or when an event (like a clutch release) is over before the next GPS fix even arrives.

- Propose an approach to make the detector robust to this GPS cadence mismatch while still running in real time on a stream of incoming samples. Your answer can use interpolation, event-driven windowing, sanity-checking against physical limits, or another approach of your choice — briefly justify the choice.

- Write pseudocode (10–15 lines) for a streaming version of your approach — i.e. one that processes samples one at a time as they arrive, rather than looping over a complete stored array.

## Q5. Choosing a model for on-device detection

You need to classify short driving-sensor windows in real time, on-device (a mid-range Android phone), into one of: Normal Driving, Harsh Braking, Harsh Acceleration, Pothole/Bump, or Clutch Release. You are choosing between three approaches: (a) hand-tuned rule-based thresholds, (b) a Random Forest / XGBoost classifier on engineered features, (c) an LSTM operating directly on the raw sensor sequence.

- Which would you pick as a first version, and why? Consider labeled-data availability, inference latency, interpretability, and how easy it is to debug in the field.

- Clutch Release is shorter and jitterier than the other events. Does that change which approach you'd pick just for that one class, or how you'd window the data for it? Explain briefly.

- Name two evaluation metrics (beyond accuracy) you would report before shipping this model, and explain briefly why each matters for this use case.

## Bonus (optional): run it on a full trip

Everything above uses a short excerpt simplified to 10 Hz so it's easy to read by eye. Also attached is driving_session_30min_50Hz.csv — a 30-minute, 90,000-row log at the real 50 Hz rate, same mount and same columns as before. It's not required, and skipping it will not count against you — but if you'd like to show your Q5 approach actually working, not just described, you're welcome to implement a simple version of it and run it against this file.

- Report roughly how many Harsh Braking, Harsh Acceleration, Pothole/Bump, and Clutch Release events your approach flags, plus anything you noticed while tuning it (false positives, a threshold that needed adjusting, etc.). There's no hidden “correct” count we're grading you against — we're interested in your approach and code, not an exact number.

- Note: this file has events packed in much more densely than a real drive would (so there's enough to test against) — don't read its event frequency as realistic; Q3's 1.8% figure is the realistic one.

Submission: reply with your answers as a document or notebook, plus any code you wrote for Q2/Q4/the bonus. Partial or well-reasoned answers are fine — we're evaluating how you think, not just final numbers.